DUAL CORE EDGE COMPUTING FRAMEWORK

Power Factor Control & Load Demand Management

─────────────────────────────────────────────

HARDWARE CONNECTION GUIDE — VERSION 2 (REVISED)

Incorporates: Updated Relay Channel Assignment | Corrected Sensor 2 & 3 Wiring | Renewable Source Distribution Fix

# 1. WHAT CHANGED FROM V1 — QUICK REFERENCE

| Item | V1 (Previous) | V2 (Revised — Correct) |
| --- | --- | --- |
| Relay CH1 | Motor 1 Load | Capacitor Bank 1 (PF Correction) |
| Relay CH2 | Motor 2 Load | Capacitor Bank 2 (PF Correction) |
| Relay CH3 | Capacitor Bank 1 | Solar Source Switch (12V Adapter 1) |
| Relay CH4 | Capacitor Bank 2 | Wind Source Switch (12V Adapter 2) |
| GPIO 25 | Motor 1 Control | Capacitor Bank 1 Control (unchanged GPIO) |
| GPIO 26 | Motor 2 Control | Capacitor Bank 2 Control (unchanged GPIO) |
| GPIO 27 | Cap Bank 1 | Solar Source Relay (unchanged GPIO) |
| GPIO 14 | Cap Bank 2 | Wind Source Relay (unchanged GPIO) |
| CS1 (GPIO 35) | Line 1 Current | Overall AC Mains Current (230V side) |
| CS2 (GPIO 32) | Line 2 Current | Combined Load 1 + Load 2 Current |
| CS3 (GPIO 33) | Mains Total | Combined Solar + Wind Generation Current |
| Sensor 3 Wiring | Solar+ to T1, Wind+ to T2 (WRONG — short circuit) | Both sources merge to 1 wire through sensor (CORRECT) |

⚠  NOTE: The ESP32-to-relay wire connections (GPIO to relay IN1–IN4) do NOT change. Only the load-side terminal connections of the relay are updated.

# 2. WHY YOUR SENSOR 3 CONNECTION WAS WRONG — EXPLAINED

## 2.1 How a Current Sensor Works (ACS712 / ZMCT103C)

A current sensor has exactly ONE current path — a conductor passes through it in one direction. Terminal 1 is where current enters, Terminal 2 is where it exits. There are no separate inputs. The sensor measures the total current flowing through this single conductor.

## 2.2 What Your Previous Wiring Did

You connected:

- Solar (+) → Terminal 1 of Sensor 3

- Wind (+) → Terminal 2 of Sensor 3

This means Solar (+) and Wind (+) were connected to opposite ends of the same conductor inside the sensor. This does NOT add them — it connects them directly together through the sensor shunt, effectively short-circuiting both adapters against each other. Since both adapters are at 12V, the potential difference across the sensor is 0V and you see only 12V (or unstable voltage) on the combined output — no usable current reading is possible.

## 2.3 The Correct Mental Model

Think of the sensor like a clamp meter. You don't put two wires into a clamp meter from opposite sides — you pass a single combined wire through the clamp. The current sensor works the same way:

WRONG:  Solar+ ─────> [T1 === T2] <───── Wind+   (short circuit!)

CORRECT: Solar+ ──┐

├──> JUNCTION WIRE ──> [T1 → SENSOR → T2] ──> Load / Bus

Wind+  ──┘

Both source positives join at a single junction node (one wire). That single combined wire carries the total current from both sources. This combined wire is what you pass through Sensor 3. The sensor then correctly measures the total generation current.

⚠  NOTE: Current sensors measure current through a conductor, not voltage between two points. Always connect two sources in parallel FIRST (join their positive wires), then pass the single combined positive through the sensor.

# 3. SENSOR 2 — MEASURING BOTH LOADS (MOTOR 1 + MOTOR 2)

## 3.1 The Same Logic Applies to Sensor 2

Sensor 2 (GPIO 32) measures the combined current drawn by Load 1 (Motor 1) and Load 2 (Motor 2). Both motors are powered from the same DC supply rail. Their combined current flows through a single feed wire before it branches to each motor relay. You must pass that single combined feed wire through Sensor 2.

## 3.2 Correct Sensor 2 Wiring

The correct placement is BEFORE the branch point — on the common supply line that feeds both relays (CH3 and CH4 in your new assignment), before the wire splits to each relay's COM terminal.

24V Bus (+) ──> [T1 → SENSOR 2 → T2] ──> JUNCTION

│

┌───────────┴───────────┐

│                       │

Relay CH3 COM          Relay CH4 COM

(Motor 1)              (Motor 2)

This way, whenever either or both motors draw current, the total flows through Sensor 2 and is correctly measured.

⚠  NOTE: If you need INDIVIDUAL motor currents separately, you would need two sensors — one per relay output. But since your design uses CS2 for total load current, the above single-sensor placement on the combined feed is correct and sufficient.

| Sensor 2 Terminal | Connects To | Wire Description |
| --- | --- | --- |
| Terminal 1 (IN) | 24V Bus positive (common motor supply feed) | Combined feed wire entering sensor |
| Terminal 2 (OUT) | Junction point → Relay CH3 COM and Relay CH4 COM | Wire exits sensor and splits to both relays |

# 4. SENSOR 3 — CORRECT WIRING FOR SOLAR + WIND MEASUREMENT

## 4.1 Step-by-Step Connection

Follow these steps in sequence for Sensor 3:

- Create a junction node: Take the positive wire from 12V Adapter 1 (Solar) and the positive wire from 12V Adapter 2 (Wind). Twist or solder them together to form a single combined positive wire at one junction point. This junction is where both sources merge.

- Pass through Sensor 3: Connect this combined junction wire into Terminal 1 of Sensor 3 (current entry). The wire exits from Terminal 2 of Sensor 3.

- Branch AFTER the sensor: The wire exiting Terminal 2 of Sensor 3 now splits into two branches — one going to Relay CH3 COM terminal (Solar switch) and one going to Relay CH4 COM terminal (Wind switch).

- GND connection: GND of Adapter 1 and GND of Adapter 2 both connect to Common GND directly (no sensor needed on return path).

12V Adapter 1 (+) ──┐

├──> Combined(+) wire

12V Adapter 2 (+) ──┘       │

│

[T1 → CS3 → T2]    (Sensor 3 measurement)

│

┌────────┴────────┐

│                 │

Relay CH3 COM      Relay CH4 COM

(Solar switch)     (Wind switch)

│                 │

Relay CH3 NO       Relay CH4 NO

│                 │

──> DC Load Bus  ──> DC Load Bus  (to motors/ESP32)

| Sensor 3 Terminal | Connects To | What It Carries |
| --- | --- | --- |
| Terminal 1 (IN) | Junction of Solar(+) and Wind(+) wires | Combined current from both sources entering sensor |
| Terminal 2 (OUT) | Junction → Relay CH3 COM and Relay CH4 COM | Combined current exiting sensor, then distributed to both relay switches |

⚠  NOTE: Because the sensor is on the combined line BEFORE the relay branches, it always measures total generation current — whether only solar is active (CH3 relay closed), only wind (CH4 closed), or both together. This is the correct and complete measurement.

# 5. HOW RELAY CH3 & CH4 SWITCH SOLAR AND WIND TO THE LOAD BUS

## 5.1 Purpose of CH3 and CH4 Relays

Relay CH3 and CH4 are not just for measurement — they actively switch each renewable source onto the DC load bus. This allows the ESP32 to selectively enable solar, wind, or both, depending on availability and load demand. If AC mains is present, both can remain open. When mains fails, the ESP32 closes these relays to connect the backup sources.

## 5.2 Relay Load-Side Terminal Connections (Revised)

| Relay CH | GPIO Control | COM Terminal | NO Terminal | NC Terminal | Function |
| --- | --- | --- | --- | --- | --- |
| CH1 | GPIO 25 | AC Line (L) after MCB | Cap Bank 1 (terminal 1) | Not used | Switch Cap 1 across AC load for PF correction |
| CH2 | GPIO 26 | AC Line (L) after MCB | Cap Bank 2 (terminal 1) | Not used | Switch Cap 2 across AC load for PF correction |
| CH3 | GPIO 27 | CS3 T2 output (combined source wire) | DC Load Bus (+) | Not used | Connect Solar+Wind bus to load bus when active |
| CH4 | GPIO 14 | CS3 T2 output (combined source wire, same junction) | DC Load Bus (+) | Not used | Redundant/selective switch — can isolate wind or solar individually if wired separately |

## 5.3 Alternative: Individual Source Switching (Recommended Enhancement)

If you want Relay CH3 to switch ONLY Solar and Relay CH4 to switch ONLY Wind independently (so the ESP32 knows which source is contributing), wire them separately BEFORE the Sensor 3 junction:

12V Adapter 1 (+) ──> Relay CH3 COM ──> Relay CH3 NO ──┐

├──> [T1 → CS3 → T2] ──> DC Load Bus(+)

12V Adapter 2 (+) ──> Relay CH4 COM ──> Relay CH4 NO ──┘

In this layout: each adapter first passes through its own relay (individual switching control), then both relay outputs join and together pass through Sensor 3 (combined measurement), then go to the DC load bus. This gives you the best of both worlds — individual control + combined current measurement.

✔  FIX: This individual-switching layout is the recommended connection for your project as it enables the ESP32 to select, prioritize, and monitor each renewable source independently.

| Connection Point | From | To | Purpose |
| --- | --- | --- | --- |
| Solar relay input | 12V Adapter 1 (+) | Relay CH3 COM | Solar source entry |
| Solar relay output | Relay CH3 NO | Junction A (pre-sensor) | Solar switched output |
| Wind relay input | 12V Adapter 2 (+) | Relay CH4 COM | Wind source entry |
| Wind relay output | Relay CH4 NO | Junction A (pre-sensor) | Wind switched output |
| Sensor 3 entry | Junction A | CS3 Terminal 1 | Combined generation current IN |
| Sensor 3 exit | CS3 Terminal 2 | DC Load Bus (+) | Combined generation current OUT to bus |
| GND return | Adapter 1 GND + Adapter 2 GND | Common GND bus | Return path for both sources |

# 6. SENSOR 1 — OVERALL AC MAINS CURRENT (230V SIDE)

Sensor 1 (GPIO 35) measures the total current drawn from the 230V AC mains. It must be placed on the Live (L) wire after the MCB, before any branch to loads or the AC-DC converter.

MCB Output (L) ──> [T1 → CS1 → T2] ──> Distribution rail

│

┌─────────────────────┼─────────────────────┐

│                     │                     │

AC-DC Converter        Relay CH1 COM         Relay CH2 COM

(system power)         (Cap Bank 1)          (Cap Bank 2)

| CS1 Terminal | Connects To | Notes |
| --- | --- | --- |
| Terminal 1 (IN) | MCB Live output (L wire) | Entry of AC live wire into sensor |
| Terminal 2 (OUT) | 230V AC distribution rail | All AC loads (converter + relay COMs for caps) fed from here |
| VCC | 3.3V from ESP32 | Sensor supply |
| GND | Common GND |  |
| Signal OUT | ESP32 GPIO 35 | ADC measurement |

⚠  NOTE: For ZMCT103C (CT type): only the Live (L) wire passes through the CT ring. Neutral does NOT pass through the CT. For ACS712: only the Live wire passes through the module's current path pins (IP+ and IP-). Never pass both Live and Neutral through the same current sensor — the fields cancel and you get zero reading.

# 7. MASTER REVISED HARDWARE CONNECTION TABLE

## 7.1 ESP32 GPIO Allocation (Unchanged from V1)

| GPIO | Direction | Connected To | Function |
| --- | --- | --- | --- |
| GPIO 21 | Output (I2C SDA) | OLED Display SDA | I2C Data |
| GPIO 22 | Output (I2C SCL) | OLED Display SCL | I2C Clock |
| GPIO 34 | Input ADC | ZMPT101B OUT | AC Voltage (230V) measurement |
| GPIO 35 | Input ADC | CS1 Signal OUT | Overall AC mains current |
| GPIO 32 | Input ADC | CS2 Signal OUT | Combined Motor 1 + Motor 2 load current |
| GPIO 33 | Input ADC | CS3 Signal OUT | Combined Solar + Wind generation current |
| GPIO 25 | Output | Relay IN1 | Capacitor Bank 1 (PF Correction) |
| GPIO 26 | Output | Relay IN2 | Capacitor Bank 2 (PF Correction) |
| GPIO 27 | Output | Relay IN3 | Solar Source Switch |
| GPIO 14 | Output | Relay IN4 | Wind Source Switch |
| 3V3 | Power Out | CS1, CS2, CS3 VCC, ZMPT VCC, OLED VCC | Sensor power rail |
| GND | Common GND | All module GNDs | Ground reference |
| VIN | Power In | 5V from LM7805 | Board power |

## 7.2 Relay Module — Revised Load Side Connections

| Channel | GPIO | COM Terminal Connects To | NO Terminal Connects To | Function |
| --- | --- | --- | --- | --- |
| CH1 | GPIO 25 | AC Live (L) from CS1 output rail | Cap Bank 1 — Terminal 1 | PF Correction Cap 1 |
| CH2 | GPIO 26 | AC Live (L) from CS1 output rail | Cap Bank 2 — Terminal 1 | PF Correction Cap 2 |
| CH3 | GPIO 27 | 12V Adapter 1 (+) [Solar] | Junction A (pre-CS3) | Solar source switch |
| CH4 | GPIO 14 | 12V Adapter 2 (+) [Wind] | Junction A (pre-CS3) | Wind source switch |

## 7.3 Capacitor Banks — Load Side (Unchanged)

| Capacitor | Terminal 1 (L side) | Terminal 2 (N side) | Switched by |
| --- | --- | --- | --- |
| 10µF AC Cap 1 | Relay CH1 NO output | Neutral (N) rail | GPIO 25 / Relay CH1 |
| 10µF AC Cap 2 | Relay CH2 NO output | Neutral (N) rail | GPIO 26 / Relay CH2 |

# 8. FIRMWARE CHANGES NEEDED FOR REVISED HARDWARE

## 8.1 Relay Logic Update

The GPIO pins are unchanged but the meaning of each relay in firmware must be updated:

| GPIO | Old Firmware Role | New Firmware Role |
| --- | --- | --- |
| GPIO 25 (Relay CH1) | motorRelay1 — ON/OFF Motor 1 | capRelay1 — PF Correction Cap 1 |
| GPIO 26 (Relay CH2) | motorRelay2 — ON/OFF Motor 2 | capRelay2 — PF Correction Cap 2 |
| GPIO 27 (Relay CH3) | capRelay1 — PF Correction | solarRelay — Enable Solar source |
| GPIO 14 (Relay CH4) | capRelay2 — PF Correction | windRelay — Enable Wind source |

## 8.2 ADC Channel Role Update

| ADC GPIO | Old Role | New Role |
| --- | --- | --- |
| GPIO 35 (CS1) | Load Line 1 / Motor 1 Current | Total AC Mains Current (230V incoming) |
| GPIO 32 (CS2) | Load Line 2 / Motor 2 Current | Combined Motor 1 + Motor 2 Load Current |
| GPIO 33 (CS3) | Total mains current | Combined Solar + Wind Generation Current |

## 8.3 Key Logic Rules for Core 0

- AC Mains present → GPIO 27 and GPIO 14 remain LOW (relays open, adapters disconnected from bus)

- AC Mains lost (detected via ZMPT101B reading drops to zero) → GPIO 27 HIGH (solar relay closes), GPIO 14 HIGH (wind relay closes)

- PF < 0.95 → GPIO 25 HIGH (Cap 1 connects) → monitor PF

- PF still < 0.92 → GPIO 26 HIGH (Cap 2 also connects) → monitor PF

- PF > 0.98 → GPIO 25 and GPIO 26 LOW (caps disconnect, avoid leading PF)

- CS2 reading > overload threshold → both motor loads can be shed in backup mode

- CS3 reading = 0 while backup mode → no generation present, alert on OLED

# 9. FULL SYSTEM WIRING DIAGRAM (TEXT)

════════════════════════════════════════════════════════════════

230V AC MAINS SIDE

════════════════════════════════════════════════════════════════

[Wall L] ─> [MCB] ─> [CS1 T1→T2] ─> AC Distribution Rail

│

┌─────────────────────────────┼──────────────┐

│                             │              │

[AC-DC Conv L]              [Relay CH1 COM]  [Relay CH2 COM]

│                             │              │

[AC-DC Conv N]<─[Wall N]    [CH1 NO]─[Cap1]  [CH2 NO]─[Cap2]

│                             │              │

12V DC Out (+)             Cap1 T2─>[N]     Cap2 T2─>[N]

│

════════════════════════════════════════════════════════════════

DC POWER BUS SIDE

════════════════════════════════════════════════════════════════

AC-DC 12V ──[D1]──┐

Adapter1(+)──[D2]──┼──> 12V Common Bus ──> LM7805 ──> 5V

Adapter2(+)──[D3]──┘

│

┌──────────────────┼──────────────────┐

[ESP32]         [Relay VCC]        [Sensors+OLED]

════════════════════════════════════════════════════════════════

RENEWABLE SOURCE SWITCHING

════════════════════════════════════════════════════════════════

Adapter1(+)──> Relay CH3 COM ──> CH3 NO ──┐

├─>[T1→CS3→T2]──> DC Load Bus(+)

Adapter2(+)──> Relay CH4 COM ──> CH4 NO ──┘

════════════════════════════════════════════════════════════════

LOAD SIDE (MOTORS on DC BUS)

════════════════════════════════════════════════════════════════

DC Load Bus(+) ─>[T1→CS2→T2]─> Junction ─┬─> Relay CH3_M COM

(Note: CH3 and CH4 below = Motor relays   └─> Relay CH4_M COM

if you use separate motor relays,

or control motors via separate relay board)

Motor 1: Relay NO ──> Motor1(+), Motor1(–)──> GND

Motor 2: Relay NO ──> Motor2(+), Motor2(–)──> GND

⚠  NOTE: Since you now use CH1 and CH2 for capacitors, and CH3 and CH4 for source switching, motor ON/OFF control needs to be handled by a separate relay board or by combining motor control with the source relay logic in firmware (motors only run when source relay is closed).

# 10. TROUBLESHOOTING GUIDE

| Symptom | Likely Cause | Fix |
| --- | --- | --- |
| Only 12V seen on combined source bus | Solar+ and Wind+ shorted through sensor terminals | Rewire: join both (+) FIRST at junction, then single wire through sensor |
| CS3 always reads zero | Sensor placed after relay (relay is open, no current flows) | Place sensor BEFORE relay on source feed wire, or close relay first for test |
| CS2 reads only one motor current | Sensor placed on individual motor wire instead of combined feed | Move sensor to the common 24V supply wire before it splits to both relays |
| Relay chatters rapidly | PF threshold too tight in firmware hysteresis | Add hysteresis band: cap ON below 0.92, cap OFF above 0.98 |
| System dies when mains removed | Diode OR failover not wired, or LDO drops out | Check D1/D2/D3 diode circuit, verify 12V bus remains stable on adapters |
| OLED shows garbage | I2C address mismatch | Run I2C scanner, set correct address (0x3C or 0x3D) in code |
| PF reading stuck at 1.0 | Voltage and current sensors not in phase or calibration offset wrong | Check ADC sampling rate, ensure V and I sampled at same timestamp |

Hardware Guide Version 2 | Revised per updated relay assignment and sensor connection corrections

All AC high-voltage work must be performed under qualified supervision | Follow IEC/IS safety standards